Most Important Comparisons of Drinking Water Specifications

| Parameters | WHO Guidelines of Drinking Water Quality 1999 | USA USEPA National Primary Drinking Water Regulations | EU European Communities Drinking Water Regulations 2000 |
| --- | --- | --- | --- |
| Colour | 15 TCU* | 15 Colour Units | Acceptable to consumers and no abnormal change |
| Odour | No guideline | 3 Threshold Odor Number | Acceptable to consumers and no abnormal change |
| Taste | No guideline | ─ | ─ |
| Turbidity | 5 NTU* | NA | Acceptable to consumers and no abnormal change |
| Chlorine | No guideline | 4 mg/L as Cl2 Maximum Residual Disinfectant Level | Not mentioned |
| PH | No guideline | 6.5 – 8.5 | 6.5-9.5 |
| TDS | 1000 mg/L* | 500 mg/L | Not mentioned |
| Total Hardness as CaCO3 | No guideline | Not mentioned | Not mentioned |
| Calcium as Ca | No guideline | Not mentioned | Not mentioned |
| Magnesium as Mg | No guideline | Not mentioned | Not mentioned |
| Alkalinity | No guideline | Not mentioned | Not mentioned |
| Chloride | 250 mg/L * | 250 mg/L | 250 mg/L |
| Fluoride | 1.5 mg/L | 4.0 mg/L | 1 mg/L |
| Nitrate | 50 mg/L as NO3 | 10 as N2 / 44.44 as NO3 | 50 mg/L |
| Nitrite | 3(P) | 1 as N2 | 0.50 mg/L |
| Sulphate | 250 mg/L* | 250 mg/L | 250 mg/L |
| Iron | 0.3 mg/L * | 0.3 mg/L | 0.2 mg/L |
| Ammonia | 1.5mg/L* | Not mentioned | 0.3 mg/L |
| Aluminum | 0.2mg/L* | 0.05 – 0.2 mg/L | 0.2 mg/L |
| Arsenic | 0.01 mg/L (P) | 0.01 mg/L | 0.05 mg/L |
| Antimony | 0.005 mg/L (P) | 0.006 mg/L | 0.01 mg/L |
| Asbestos | No guideline | 7 MFL | Not mentioned |
| Barium | 0.7 mg/L | 2 mg/L | Not mentioned |
| Beryllium | No guideline | 0.004 mg/L | Not mentioned |
| Brass | No guideline | Not mentioned | Not mentioned |
| Boron | 0.3 mg/L | Not mentioned | 1 mg/L |
| Bromate | 0.025 mg/L (P) | Zero | 0.01 mg/L |
| BOD | No guideline | Not mentioned | Not mentioned |
| Cadmium | 0.003 mg/L | 0.005 mg/L | 0.005 mg/L |
| Chromium | 0.05 mg/L (P) | 0.1 mg/L | 0.05 mg/L |
| Copper | 2 mg/L (P) | 1.3 mg/L | Zero |
| Cyanide | 0.07 mg/L | 0.2 mg/L | 0.05 mg/L |
| Dissolved Oxygen | No guideline | Not mentioned | Not mentioned |
| Hydrogen sulphide | No guideline | Not mentioned | Not mentioned |
| Lead | 0.01 mg/L | 0.015 mg/L | 0.01 mg/L |
| Manganese | 0.5 mg/L (P) | 0.05 mg/L | 0.05 mg/L |
| Mercury | 0.001 mg/L | 0.002 mg/L | 0.001 mg/L |
| Molybdenum | 0.07 mg/L | Not mentioned | Not mentioned |
| Nickel | 0.02 mg/L | Not mentioned | Zero |
| Selenium | 0.01 mg/L | 0.05 mg/L | 0.01 mg/L |
| Silver | No guideline | 0.1 mg/L | Not mentioned |
| Sodium | 200 mg/L | Not mentioned | 200 mg/L |
| Potassium | Not mentioned | Not mentioned | Not mentioned |
| Thallium | Not mentioned | 0.0005 mg/L | Not mentioned |
| Zinc | 3 mg/L | 5 mg/L SS | Not mentioned |
| Anionic detergents | Not mentioned | ─ | Not mentioned |
| Poly nuclear aromatic hydrocarbon | Not mentioned | Zero | 0.0001 mg/L |
| Poly chlorinated bi phenyl | Not mentioned | Not mentioned | Not mentioned |
| Phenolic compounds | Not mentioned | Zero | Not mentioned |
| Total Trihalomethanes | ─ | Zero | 0.1 mg/L |
| Trihalomethanes (bromoform) | 0.1 mg/L | Not mentioned - | Not mentioned - |
| Trihalomethanes (chloroform) | 0.2 mg/L | Not mentioned | Not mentioned |
| Acrylamide | 0.0005 mg/L | Zero | 0.0001 mg/L |
| Benzene | 0.01 mg/L | Zero | 0.001 mg/L |
| Benzo pyrene | 0.0007 mg/L | Zero | 0.00001 mg/L |
| 1,2 dichloro ethane | 0.03 mg/L | Zero | 0.003 mg/L |
| Epichlorohydrin | 0.0004 mg/L (P) | Zero | 0.0001 mg/L |
| Tetra chloro ethane | 0.04 mg/L | Zero | 0.01 mg/L |
| Trichloroethane | 0.07 mg/L | Zero | 0.01 mg/L |
| Vinyl chloride | 0.005mg/L | Zero | 0.0005 mg/L |
| Carbon tetra chloride | 0.002 mg/L | Zero | Not mentioned |
| Dichloroethane | 0.05 mg/L | Zero | Not mentioned |
| Xylene | 0.5 mg/L | 10 | Not mentioned |
| Toluene | 0.7 mg/L | 1 mg/L | Not mentioned |
| Carbofuran | Not mentioned | 0.004 mg/L | Not mentioned |
| Chlordane | Not mentioned - | Zero | Not mentioned |
| Chlorobenzene | 0.3 mg/L | 0.1 mg/L | Not mentioned |
| Ethyl benzene | 0.3 mg/L | 0.7 mg/L | Not mentioned |
| Styrene | 0.02 mg/L | 0.1 mg/L | Not mentioned |
| Dibromo chloromethane | 0.1 mg/L | Not mentioned | Not mentioned |
| Bromodichloromethane | 0.06 mg/L | Not mentioned | Not mentioned |
| Monochloramine & di & trichloramine chlorine | 3 | Not mentioned | Not mentioned |
| Dichloramine & Trichloramine chlorine | 5 | Not mentioned | Not mentioned |
| Pesticides | Not mentioned | Not mentioned | mg/L Total Pesticide 0.0005mg/L) |
| 1) Lindane | 0.002 mg/L | 0.0002 mg/L | Not mentioned |
| 2) Endrin | ─ | 0.002 mg/L | Not mentioned |
| 3) Diquate | ─ | 0.02 mg/L | Not mentioned |
| 4) Glyphosphate | ─ | 0.7 mg/L | Not mentioned |
| 5) Heptachlor | 0.00003 mg/L | Zero | Not mentioned |
| 6)Heptachlorepoxide | 0.00003 mg/L | Zero | Not mentioned |
| 7) Picloram | ─ | 0.5 mg/L | Not mentioned |
| 8) Simazine | 0.002 mg/L | 0.004 mg/L | Not mentioned |
| 9) Toxaphene | ─ | Zero mg/L | Not mentioned |
| 10) 2,4,5 TP (Silvex) | ─ | 0.05 mg/L | Not mentioned |
| 11) Endothall | ─ | 0.1 mg/L | Not mentioned |
| 12) Alachlor | 0.02 mg/L | Zero | Not mentioned |
| 13) Atrazine | 0.002 mg/L | 0.003 mg/L | Not mentioned |
| 14) DDT | 0.002 mg/L | Not mentioned | Not mentioned |
| 15) Aldrine/Dieldrin |  | Not mentioned | Not mentioned |
| 16) Endosulphan |  | Not mentioned | Not mentioned |
| 17) Chlorpyriphos |  | Not mentioned | Not mentioned |
| 18) Malatheon |  | Not mentioned | Not mentioned |
| 19) 2,4 D | 0.03 mg/L | 0.07 mg/L | Not mentioned |
| 20) Isoproturon | 0.009 mg/L | Not mentioned | Not mentioned |
| 21) Alpha HCH |  | Not mentioned | - |
| 22) Beta HCH | Not mentioned | Not mentioned | Not mentioned |
| 23) Delts HCH | Not mentioned | Not mentioned | Not mentioned |
| 24 Monocrotophos | Not mentioned | Not mentioned | Not mentioned |
| 25) Ethion | Not mentioned | Not mentioned | Not mentioned |
| 26) Phorate | Not mentioned | Not mentioned | Not mentioned |
| 27) Butachlor | Not mentioned | Not mentioned | Not mentioned |
| Radioactive material Alpha, Beta, Radium 226 Radium 228 & Uranium | 0.1 Bq/L for Alpha and 1 Bq/L for Beta | Zero | Tritium-100 Bq/Lb |
| Bacteria – Coliforms | Zero | Zero | Zero |
| Bacteria – E.Coli | Zero | Zero | Zero |
| Enterococci | No guideline | No guideline | Zero |
| HPC-colony | Not mentioned | Not mentioned | Zero |
| Clostridium perfringens (spores) | Not mentioned | Not mentioned | Zero |
| Virus | No guideline- | Zero | Not mentioned |
| Protozoa- Cryptosporidium | No guideline | Zero | Not mentioned |
| Protozoa-Gairdia | No guideline | Zero | Not mentioned |